# Section 4:  Main Components Assembly

This section walks you through the assembly of the two most foundational components of Board 1: the Grove Base Hat and the Pimoroni Enviro+ HAT. These two boards are the reason the rest of the build is beginner-friendly — together, they turn the Raspberry Pi's complex GPIO header into a simple, plug-and-play sensor platform. Understanding them thoroughly before moving on will make every subsequent section easier.

| HOW THIS SECTION WORKS | Each component is introduced with: what it is and why it matters → what needs to be done before plugging it in → how to connect it → what data to expect from it → how it fits into the broader system → maintenance needed → any conflicts to watch for. Work through each component fully before moving to the next. |
| --- | --- |

## 4.1  Grove Base Hat for Raspberry Pi

| Grove Base Hat for Raspberry Pi Seeed Studio — 40-pin GPIO expansion with Grove port system | Board 1 — Core |
| --- | --- |

### What It Is and Why It Matters

The Grove Base Hat is an add-on board that plugs directly onto the Raspberry Pi's 40-pin GPIO header. Its job is to make connecting sensors and modules much simpler by converting the Pi's raw electrical pins into standardised Grove ports — small, colour-coded 4-pin sockets that accept Grove cables with a single click.

Without the Grove Base Hat, connecting each sensor to the Pi requires identifying individual pin numbers, selecting the right voltage levels, and wiring each connection manually — a process prone to mistakes and easily damaged components. With the Grove Base Hat, you simply identify which type of port a sensor needs (I2C, UART, digital, analog, or PWM) and plug in the cable. The hat handles all the electrical routing.

| Grove Port Type | Count on Hat | What Connects Here | GPIO Pins Used |
| --- | --- | --- | --- |
| I2C | 3 ports | All I2C sensors: Enviro+ internals, IMU, GPS, Arducam, UPS HAT | GPIO 2 (SDA), GPIO 3 (SCL) — all three ports share the same bus |
| UART | 1 port | NEO-M8M GPS module | GPIO 14 (TX), GPIO 15 (RX) |
| PWM | 1 port | Motor driver enable pins (ENA/ENB) | GPIO 12 (PWM0), GPIO 13 (PWM1) |
| Digital | 6 ports | Motor direction pins, miscellaneous GPIO control | D5, D16, D18, D22, D24, D26 |
| Analog | 4 ports | Any analog Grove sensor (via built-in 12-bit ADC) | Internal STM32 MCU → I2C to Pi |
| SWD | 1 port | Firmware update for the Hat's MCU — not used in normal operation | Internal only |

### Built-In ADC — Why It Matters

The Raspberry Pi has no built-in analog-to-digital converter — it cannot read analog voltages directly. The Grove Base Hat solves this with an onboard STM32 microcontroller that provides a 12-bit, 8-channel ADC. Any analog Grove sensor plugged into the four analog ports is read by this ADC and the result is passed to the Pi over I2C. This means the Pi can effectively read analog sensors without any additional hardware.

### Before You Plug It In

| IMPORTANT | The Grove Base Hat operates at 3.3V. Never connect a 5V Grove module to any Grove port on this hat — it will damage both the hat and the Raspberry Pi. Check the voltage specification of every Grove module before connecting it. All modules used in EnviroSat are 3.3V compatible. |
| --- | --- |

| Preparation Step | What to Do | Why |
| --- | --- | --- |
| Check Pi GPIO header | Inspect all 40 pins for bends or damage before attaching the hat | A bent pin can short two pins together and damage the Pi when the hat is pressed down |
| Enable I2C | Boot Pi, open Terminal, run: sudo raspi-config → Interface Options → I2C → Enable | I2C must be enabled in software before any I2C Grove module will respond |
| Enable SPI | In raspi-config → Interface Options → SPI → Enable | SPI is needed for LoRa and the Enviro+ LCD |
| Disable serial console | raspi-config → Interface Options → Serial Port → Shell: NO → Hardware: YES | The Pi uses GPIO 14/15 for a debug console by default — this must be freed for GPS |
| Power off Pi | Run sudo shutdown -h now and wait for the green LED to stop blinking before proceeding | Never attach or remove HATs while the Pi is powered — always power off first |

### Assembly Steps

| STEP 1 | Position the Hat Hold the Grove Base Hat above the Raspberry Pi, aligning the 40-pin female header on the underside of the hat with the 40-pin male header on the Pi. The USB ports and ethernet port of the Pi should face the same direction as the Grove port labels on the Hat. Do not press down yet — just hold it in position and visually confirm all pins are aligned. |
| --- | --- |

| STEP 2 | Press Down Evenly Using both thumbs, press down gently but firmly on the corners of the Hat — not the centre. Apply even pressure so the Hat slides down squarely. You should feel and hear a small click as each pin row seats. Check that the gap between the Hat and the Pi is uniform all the way around — no one side should sit higher than the other. Do not rock the Hat — if it is not going on straight, lift it fully off and start again. |
| --- | --- |

| STEP 3 | Verify Seating Visually inspect all four sides of the 40-pin header connection. No pins should be visibly bent outward or protruding at an angle. The Hat should sit flat and parallel to the Pi board with a gap of approximately 8mm. |
| --- | --- |

| STEP 4 | Secure with Standoffs Attach M2.5 standoffs through the mounting holes on the corners of the Grove Base Hat, threading down into the Pi's mounting holes. Finger-tight is sufficient — do not overtighten or you may crack the PCB. Standoffs prevent the hat from flexing and protect the GPIO connection from mechanical stress. |
| --- | --- |

| STEP 5 | Power On and Verify Power on the Pi and SSH in from your laptop. Run: sudo i2cdetect -y 1 You should see address 0x04 appear in the grid — this is the Grove Base Hat's own STM32 microcontroller confirming the hat is seated correctly and I2C is working. If 0x04 does not appear: power off, reseat the hat, and try again. |
| --- | --- |

### Expected Data from the Grove Base Hat

The Grove Base Hat itself does not produce sensor data — it is a routing board. The verification output is simply confirmation that its MCU is visible on the I2C bus at address 0x04. Any Grove sensor you subsequently connect will then appear at its own I2C address in the i2cdetect grid.

### How It Adds Up to the Whole System

Every sensor, every radio module, and every actuator in EnviroSat reaches the Raspberry Pi through the Grove Base Hat. The three I2C ports connect the environmental sensors, IMU, camera adapter, and UPS HAT. The UART port connects the GPS. The PWM port connects the motor driver speed control. The digital ports connect the motor driver direction pins. Without the Grove Base Hat, each of these connections would require individual jumper wires to specific Pi GPIO pins — a fragile, error-prone arrangement. With it, each connection is a single, click-in cable that is physically impossible to connect backwards.

### Maintenance

- Inspect the 40-pin header connection every time the system is opened — pins can work loose over time if the system is moved frequently

- Keep Grove port sockets free of dust — a soft brush or compressed air can clean them gently

- Do not bend Grove cables at sharp angles close to the socket — this stresses the connector

- If a Grove device stops being detected, reseat its cable at both ends before suspecting a hardware fault

### Conflict Handling

| GROVE I2C BUS CONFLICT | All three Grove I2C ports share the same two GPIO pins (GPIO 2 and GPIO 3). This means all I2C devices are on the same bus simultaneously. I2C is designed for this — each device has a unique address and the Pi talks to them individually using those addresses. No special handling is needed as long as every device has a different I2C address. The only risk is if two devices share the same I2C address. The EnviroSat I2C address map in Section 3.8 confirms there are no conflicts in the default configuration. |
| --- | --- |

## 4.2  Pimoroni Enviro+ HAT

| Pimoroni Enviro+ HAT All-in-one environmental sensor array with LCD display | Board 1 — Core |
| --- | --- |

### What It Is and Why It Matters

The Enviro+ is a single board that provides six different environmental sensors plus a colour LCD display, all in one package that plugs directly onto the Pi's 40-pin header on top of the Grove Base Hat. Instead of buying, wiring, and individually configuring six separate sensor boards, you get all of them in a single unit with a single library.

| Sensor | I2C / Bus | What It Measures | Typical Output |
| --- | --- | --- | --- |
| BME280 | I2C 0x76 | Temperature, barometric pressure, relative humidity | 23.4°C / 1013 hPa / 45% RH |
| LTR-559 | I2C 0x23 | Ambient light level and proximity | 1497 Lux / proximity 0–255 |
| MICS6814 | via ADS1015 | Carbon monoxide (reducing), nitrogen dioxide (oxidising), ammonia (NH3) | Resistance values in Ohms — baseline relative |
| ADS1015 | I2C 0x49 | Analog-to-digital converter for MICS6814 | 12-bit readings for 3 gas channels |
| PMS5003 | UART | Particulate matter: PM1.0, PM2.5, PM10 | µg/m³ and particle count per 0.1L |
| MEMS Mic | I2S | Ambient noise level | Amplitude values — relative noise measurement |
| 0.96" LCD | SPI | 160×80 colour IPS display — output only | Displays any image or text rendered via Pillow library |

### Before You Plug It In — Preparation

| NOTE ON STACKING | The Enviro+ plugs onto the same 40-pin GPIO header as the Grove Base Hat. This works because the Grove Base Hat uses a stacking header — a tall header that passes all 40 pins upward for the next board to use. The Enviro+ then plugs onto the top of that stacking header. The order from bottom to top is: Raspberry Pi → Grove Base Hat → Enviro+. |
| --- | --- |

| Preparation Step | What to Do | Why |
| --- | --- | --- |
| Install Python library | Run the install script — see Step 1 below | The Enviro+ requires several Python libraries that are not pre-installed on Pi OS |
| Enable I2C and SPI | Already done for Grove Base Hat — confirm both are enabled in raspi-config | Enviro+ uses both I2C (sensors) and SPI (LCD) |
| Enable I2S | Add dtoverlay=googlevoicehat-soundcard to /boot/config.txt — only if using mic | The MEMS microphone uses the I2S audio interface |
| PMS5003 precaution | If using the particulate sensor, connect it BEFORE powering the Pi | Hot-plugging the PMS5003 cable while Pi is on causes an immediate reboot |
| LCD protective film | Peel the green-tabbed protective film off the LCD screen after assembly | The film looks like the screen is blank — easy to mistake for a fault if left on |
| Power off Pi | Always power off before attaching the Enviro+ | Same rule as Grove Base Hat — never hot-plug a HAT |

### Assembly Steps

| STEP 1 | Install the Enviro+ Python Library With the Pi running (before attaching the Enviro+ board), SSH in and run: git clone https://github.com/pimoroni/enviroplus-python cd enviroplus-python ./install.sh When prompted about virtual environment setup, select Yes. When installation completes, run: sudo reboot The install script automatically enables I2C, SPI, and disables the serial console. |
| --- | --- |

| STEP 2 | Power Off the Pi Run: sudo shutdown -h now Wait for the green activity LED on the Pi to stop blinking completely before proceeding. Disconnect the Pi's power supply. |
| --- | --- |

| STEP 3 | Attach the Enviro+ to the Stacking Header The Enviro+ has a female 40-pin header on its underside. Align it carefully over the tall stacking header pins protruding from the top of the Grove Base Hat. Press down evenly with both thumbs — same technique as attaching the Grove Base Hat. The Enviro+ should sit flat and parallel, with no pins visibly misaligned. |
| --- | --- |

| STEP 4 | Connect PMS5003 Particulate Sensor (if using) Find the small JST-style connector port on the underside of the Enviro+ board, labelled PM. Connect the PMS5003 cable — it only fits one way. Do not force it. Route the sensor cable so the PMS5003 faces outward from the board stack. This must be done BEFORE powering the Pi on. |
| --- | --- |

| STEP 5 | Power On and Run First Test Reconnect power and SSH into the Pi. Activate the virtual environment: source ~/.virtualenvs/pimoroni/bin/activate Navigate to examples: cd enviroplus-python/examples Run the weather test: python weather.py You should see temperature, pressure, and humidity readings scrolling in the terminal. If readings appear, the Enviro+ is working correctly. |
| --- | --- |

| STEP 6 | Verify All Sensors on I2C Bus Run: sudo i2cdetect -y 1 You should see all of the following addresses in the grid: 0x04 — Grove Base Hat MCU 0x23 — LTR-559 light/proximity sensor 0x49 — ADS1015 ADC (for gas sensor) 0x76 — BME280 temperature/pressure/humidity If any address is missing, check the Enviro+ is fully seated on the stacking header. The LCD (SPI) and microphone (I2S) will not appear in i2cdetect — they use different buses. |
| --- | --- |

### Expected Data from Each Sensor

| Sensor | Test Command | Expected Output | What Normal Looks Like |
| --- | --- | --- | --- |
| BME280 | python weather.py | Temperature, Pressure, Humidity printed every second | ~20–30°C indoors. Note: reads 3–8°C high due to Pi CPU heat — this is normal |
| LTR-559 | python light.py | Lux value and proximity integer | Indoors: 100–500 Lux. Cover sensor: drops to near 0. Hand nearby: proximity rises |
| MICS6814 | python gas.py | Reducing, Oxidising, NH3 resistance values in Ohms | Takes 10+ minutes to stabilise. Values drift down initially then level off — normal |
| PMS5003 | python particulates.py | PM1.0, PM2.5, PM10 in µg/m³ plus particle counts | Clean indoor air: PM2.5 under 10 µg/m³. Near a road or smoke: rises significantly |
| LCD | python lcd.py | Coloured rectangle displayed on the tiny screen | If nothing shows: peel the protective film — it looks exactly like a blank screen |
| All-in-one | python all-in-one.py | All sensor data cycling on the LCD with tap-to-switch | Tap the LTR-559 proximity sensor to cycle through different sensor views on screen |

### Temperature Correction — Important

The BME280 sensor sits close to the Pi's CPU, which generates significant heat. This means raw temperature readings are typically 3–8°C higher than actual air temperature. The Enviro+ library includes a compensated temperature example that corrects for this:

| CORRECTION | Run: python compensated-temperature.py In the script, find the line: factor = 2.25 Adjust this number up or down until the reading matches a separate thermometer held next to the sensor. Typical range is 1.5 to 3.0 depending on your specific Pi and enclosure. Once calibrated, use this corrected value in all your data logging scripts. |
| --- | --- |

### How the Enviro+ Adds Up to the Whole System

The Enviro+ is EnviroSat's primary scientific instrument. Every data packet transmitted to the ground station begins with a timestamp and a set of Enviro+ readings. The temperature, humidity, and pressure data provides the environmental context for all other measurements. The gas sensor detects air quality changes. The particulate sensor is the most mission-critical sensor for outdoor deployment, providing real-world air pollution data. The LCD serves as a local display so the system's status can be read at a glance without needing a connected laptop.

### Maintenance

- The BME280 sensor is calibrated at the factory — no periodic recalibration is needed

- The MICS6814 gas sensor has a warm-up period of 10 minutes each time power is applied — factor this into your data logging scripts by discarding the first 10 minutes of gas readings after boot

- The PMS5003 has an internal fan — if it becomes very dusty, it can be gently cleaned by blowing compressed air through the inlet

- The LCD does not require maintenance but will burn-in if the same image is displayed for very long periods — cycle the display content or blank it when not in use

- The protective film on the LCD should be left on during assembly and storage — only peel it when you are ready for final deployment

### Conflict Handling

| Potential Conflict | Description | Resolution |
| --- | --- | --- |
| SPI bus — LCD vs LoRa | The Enviro+ LCD uses SPI (CE0, GPIO 8) and LoRa also uses SPI. Both cannot transmit simultaneously. | The LoRa library and LCD library each assert their own CS line. In software, never write to the LCD while a LoRa transmission is in progress. Use a simple flag variable in your Python script to prevent overlap. |
| UART — PMS5003 vs GPS | The PMS5003 particulate sensor also uses UART internally (connected to the Enviro+ board directly, not the Grove UART port). It uses a separate internal UART channel — it does NOT conflict with the GPS on GPIO 14/15. | No action required — these are different UART interfaces. The PMS5003 uses the Pi's secondary UART, not the primary one used by the GPS Grove port. |
| I2S microphone vs PWM audio | The MEMS microphone uses I2S. The Pi's hardware PWM also overlaps with the I2S clock on some models. | The MEMS mic is optional. If you are not using it, do not enable the I2S dtoverlay. This avoids any potential conflict with PWM-based motor control. |
| Temperature reading high | CPU heat inflates BME280 temperature readings by 3–8°C. | Apply the factor = 2.25 compensated-temperature correction described above. Calibrate against a reference thermometer. |

## 4.3  How Grove and Enviro+ Simplify the Entire Build

It is worth pausing here to appreciate what these two boards have achieved for the rest of the build. Before you have connected a single additional component, you already have:

| Capability Already Available | How It Was Achieved |
| --- | --- |
| Temperature, pressure, humidity measurement | BME280 on Enviro+ — no wiring, no calibration, no extra hardware |
| Light level and proximity sensing | LTR-559 on Enviro+ — same |
| Air quality / gas sensing | MICS6814 + ADS1015 on Enviro+ — same |
| Particulate matter measurement (PM2.5, PM10) | PMS5003 on Enviro+ — single cable connector |
| Local colour display | 0.96" LCD on Enviro+ — driven by the same library |
| 3 × I2C expansion ports | Grove Base Hat — plug any I2C sensor with a single cable |
| 1 × UART expansion port | Grove Base Hat — GPS connects here with a single cable |
| 1 × PWM expansion port | Grove Base Hat — motor speed control connects here |
| 12-bit analog input (4 channels) | Grove Base Hat built-in ADC — no extra hardware needed |
| Stable 3.3V for all connected modules | Grove Base Hat — all ports are voltage-regulated at 3.3V |

Every component added in Sections 4 onwards builds on this foundation. The Grove port system means that connecting the IMU, GPS, and motor driver each takes under a minute — find the right port type, plug in the cable, confirm the device appears in i2cdetect, and move on. This is the power of a well-designed hardware abstraction layer, and it is one of the most valuable engineering concepts you can take from building EnviroSat.

## 4.4  Board 1 Verification — What to Confirm Before Proceeding

Before moving on to Board 2 assembly, confirm every item in this checklist. Each item can be checked with a simple command or a visual inspection.

| Check | How to Verify | Expected Result |
| --- | --- | --- |
| Grove Base Hat seated | Visual inspection and: sudo i2cdetect -y 1 | Address 0x04 visible in the grid |
| I2C enabled | sudo raspi-config → Interface Options → I2C | Shows as Enabled |
| SPI enabled | sudo raspi-config → Interface Options → SPI | Shows as Enabled |
| Serial console disabled | sudo raspi-config → Interface Options → Serial Port | Shell = No, Hardware = Yes |
| Enviro+ library installed | source ~/.virtualenvs/pimoroni/bin/activate && python -c 'import bme280' | No import error |
| BME280 detected on I2C | sudo i2cdetect -y 1 | Address 0x76 visible |
| LTR-559 detected on I2C | sudo i2cdetect -y 1 | Address 0x23 visible |
| ADS1015 detected on I2C | sudo i2cdetect -y 1 | Address 0x49 visible |
| Temperature reading returns | cd enviroplus-python/examples && python weather.py | Temperature, pressure, humidity values print |
| LCD lights up | python lcd.py | Coloured box appears on the small screen |
| Temperature correction applied | python compensated-temperature.py | Calibrated temperature approximately matches room temperature |

| ALL GREEN? | If every item above shows the expected result, Board 1 is complete and fully verified. The core processing platform is working. You are ready to move on to Board 2 — power management and imaging. If any item is not passing, revisit the relevant step in 4.1 or 4.2 before continuing. |
| --- | --- |

## 4.5  What Comes Next

Board 1 gives you the brain and the sensors. The remaining sections build out the rest of the system on top of this working foundation:

| Section | What It Covers | Adds to EnviroSat |
| --- | --- | --- |
| Section 5 — Additional Instruments | LoRa radio, NRF24L01, alternatives and options | Long-range and short-range wireless communication |
| Section 6 — Subsystems | Power, command & data handling, sensors, comms, ground station | Each subsystem explained as a complete functional unit |
| Section 7 — On-Board Computer | Installing all dependencies, writing and running Python scripts | Software layer that ties all hardware together |
| Section 8 — Structure Subsystem | Physical enclosure, mounting, design requirements, testing | Mechanical integration and deployment readiness |
| Section 9 — Resources | GitHub repository, downloadable code, reference documents, videos | Everything needed for continued development |

Board 1 is your verified, working foundation. Every subsequent component in EnviroSat connects to or communicates through the Pi and Grove Base Hat stack you have just built and confirmed. Keep this board powered and accessible throughout the rest of the build — you will be returning to it frequently to run verification commands as each new module is added.