EnviroSat

Complete Wiring & Connection Prompt

For Block Diagram Illustration

| Purpose | A verified reference of every component, every connection, every pin, sourced directly from the EnviroSat documentation. Use this to build the block diagram in three panels. |
| --- | --- |
| Conflict note | Two GPIO conflicts exist and are resolved. Both resolved assignments are shown. The pre-resolution assignments are flagged clearly. |
| Panel structure | Three panels: Board 1 Core, Board 2 Power & Imaging, Board 3 Comms & Actuation. |

# 1. Component Inventory

The following 15 hardware nodes make up the complete EnviroSat satellite unit. In the block diagram, draw each as a labelled rectangle. The board assignment tells you which panel each component appears in.

| B1 | Board 1Core Processing Raspberry Pi · Grove Base Hat · Enviro+ HAT |
| --- | --- |

| Label | Component | Notes for diagram |
| --- | --- | --- |
| [A] | Raspberry Pi 3B+ | The central hub. Every other component connects through here. |
| [B] | Seeed Studio Grove Base Hat | Mounts directly on top of Pi. Converts 40-pin GPIO into Grove sockets. |
| [C] | Pimoroni Enviro+ HAT | Mounts on top of Grove Base Hat via stacking header. Show as one block with internal sensors listed: BME280 · LTR-559 · MICS6814 · ADS1015 · PMS5003 · MEMS mic · 0.96" LCD |

| B2 | Board 2Power & Imaging UPS HAT · Solar · Cameras · IMU |
| --- | --- |

| Label | Component | Notes for diagram |
| --- | --- | --- |
| [D] | Pi UPS HAT (Waveshare) + 18650 cells | Stacks on Pi via 40-pin header. Powers the Pi. |
| [E] | Solar Panel (5V USB output) | External power source. Connects via USB cable to UPS HAT. |
| [F] | Arducam UC-444 Dual Camera Adapter | Connects to Pi via 22-pin CSI ribbon cable. |
| [G] | Camera AOV5647 5MP | Connects to Arducam adapter via camera ribbon. |
| [H] | Camera BOV5647 5MP | Connects to Arducam adapter via camera ribbon. Same model as Camera A. |
| [I] | MPU-9250/6500 IMU (Grove) | Connects via Grove cable to Grove Base Hat I²C port. |

| B3 | Board 3Communications & Actuation HaLow · NRF24L01 · GPS · Motor Driver · Coil |
| --- | --- |

| Label | Component | Notes for diagram |
| --- | --- | --- |
| [J] | Heltec HT-HC01P WiFi HaLow | HT-HC01P → Debug Board (Mini Pcle) → USB cable → Pi USB port. Zero GPIO pins used. |
| [K] | 2.6 dBi SMA 915 MHz Antenna | Connects to HaLow via SMA Female–U.FL pigtail cable. |
| [L] | NRF24L01+ with Adapter Module | SPI connection via DuPont wires. Must use adapter module NOT bare module. |
| [M] | u-blox NEO-M8M GPS (Grove) | Connects via Grove cable to Grove Base Hat UART port. |
| [N] | TB6612FNG Motor Driver Shield ×2 | Direct GPIO via DuPont wires. Also has I²C Grove connection. |
| [O] | Magnetic Coil Actuator | Custom wound: 4mm iron core, AWG 32 wire, 25Ω, 200mA. Two terminals to TB6612FNG output. |

# 2. All Connections

Ten connection groups cover every wire in the system. Each group shows the exact component, port, GPIO pin, and protocol. Draw each as labelled arrows between component blocks.

## Group 1-Physical HAT Stack  (Board 1)

| STACK ORDER | Bottom to top: Raspberry Pi 3B+ → Grove Base Hat → Enviro+ HAT. All three share the 40-pin GPIO header via stacking headers. |
| --- | --- |

[A] Pi → [B] Grove Base Hat → 40-pin GPIO header (direct stack) Physical pins: all 40 pins (Pin 1 to Pin 40) Grove Base Hat uses a stacking header. all 40 pins pass upward to the Enviro+ above it

[B] Grove Base Hat → [C] Enviro+ HAT → 40-pin stacking header Physical pins: all 40 pins (passed up from Pi) Enviro+ plugs onto the tall pins protruding from the Grove Base Hat

## Group 2-I²C Bus  (shared: GPIO 2 SDA / GPIO 3 SCL)

| I²C BUS NOTE | All I²C devices share the same two wires: GPIO 2 (SDA, Pin 3) and GPIO 3 (SCL, Pin 5). Draw as a single labelled bus running from the Pi through the Grove Base Hat to all devices below. Each device is identified by its unique address. |
| --- | --- |

| From | To | I²C Address | GPIO Pins | What It Does |
| --- | --- | --- | --- | --- |
| [A] Pi | [C] Enviro+ BME280 | 0x76 | GPIO 2/3 via stack | Temperature, pressure, humidity |
| [A] Pi | [C] Enviro+ LTR-559 | 0x23 | GPIO 2/3 via stack | Light level and proximity |
| [A] Pi | [C] Enviro+ ADS1015 | 0x49 | GPIO 2/3 via stack | ADC for MICS6814 gas sensors |
| [A] Pi | [D] UPS HAT INA219 | 0x42 (or 0x36) | GPIO 2/3 via Grove I²C port | Battery voltage and current monitoring |
| [A] Pi | [F] Arducam UC-444 mux | 0x70 | GPIO 2/3 via Grove I²C port | Camera A/B selector |
| [A] Pi | [I] MPU-9250/6500 IMU | 0x68 | GPIO 2/3 via Grove I²C port | Accel, gyro, magnetometer (MPU-9250 only) |
| [A] Pi | [N] TB6612FNG driver | 0x14 | GPIO 2/3 via Grove I²C port | Motor driver I²C control (Grove variant) |
| [B] Grove Base Hat MCU | (on I²C bus) | 0x04 | Internal | STM32 ADC for analog Grove ports |

## Group 3-SPI Bus  (GPIO 9/10/11 shared; CS lines separate devices)

| Component | GPIO (BCM) | Physical Pin | SPI Signal | Notes |
| --- | --- | --- | --- | --- |
| All SPI devices | GPIO 10 | Pin 19 | MOSI (data out from Pi) | Shared |
| All SPI devices | GPIO 9 | Pin 21 | MISO (data in to Pi) | Shared |
| All SPI devices | GPIO 11 | Pin 23 | SCK (clock) | Also used as Arducam OEverify timing |
| [C] Enviro+ LCD | GPIO 8 / CE0 | Pin 24 | CSchip select | Shared with NRF24L01 CSN. Manage in software. |
| [L] NRF24L01 CSN | GPIO 8 / CE0 | Pin 24 | CSchip select | Shared with Enviro+ LCD CS. Never both active. |
| [L] NRF24L01 CE | GPIO 24 | Pin 18 | Chip Enable | RESOLVED PIN. Original table shows GPIO 17 (Pin 11)moved due to conflict. |
| [L] NRF24L01 IRQ | GPIO 25 | Pin 22 | Interrupt (optional) | Data-ready alert to Pi |
| [L] NRF24L01 VCC | Adapter 3.3V only | — | Power | NEVER connect to Pi GPIO 3.3V rail directly |
| [L] NRF24L01 GND | Pi GND | Pin 6 | Ground |  |

| NRF24L01 CE PIN CONFLICT NOTE | The NRF24L01 connections table in the document lists CE → GPIO 17 (Pin 11). GPIO 17 conflicts with the UPS HAT power-fail signal. RESOLVED: NRF24L01 CE moved to GPIO 24 (Pin 18). GPIO 24 (Pin 18) is used here. GPIO 17 is reserved for UPS HAT power-fail only. |
| --- | --- |

## Group 4-UART  (Serial)

[A] Pi → [M] NEO-M8M GPS → UART via Grove UART port on Grove Base Hat Pi GPIO 14 (Pin 8) → GPS RX (Pi transmits to GPS) Pi GPIO 15 (Pin 10) → GPS TX (GPS transmits to Pi) Baud rate: 9600 Grove UART port on Grove Base Hat REQUIREMENT: disable Pi serial login console in raspi-config first

## Group 5-USB  (HaLow WiFi)

The HaLow module uses zero GPIO pins. The full connection path is:

[J] HaLow HT-HC01P → [A] Pi USB port → USB (Mini PCIe → Debug Board → USB-A cable → Pi) HT-HC01P module (Mini PCIe form factor) → plugs into HT-HC01P Debug Board → Debug Board USB-A port → USB-A to USB-A cable → Raspberry Pi 3B+ USB-A port Pi sees this as a standard USB WiFi network adapter. No GPIO involvement at all.

[J] HaLow module → [K] SMA 915 MHz Antenna → SMA Female–U.FL pigtail HaLow module U.FL port → SMA Female–U.FL pigtail cable → SMA Male antenna ATTACH ANTENNA BEFORE POWERING MODULE. Transmitting without antenna damages the RF circuit

## Group 6-CSI Camera Ribbon

[A] Pi → [F] Arducam UC-444 → 22-pin FFC ribbon cable via Pi CSI port Pi CSI camera port (22-pin flat flexible cable) → Arducam UC-444 input Protocol: MIPI CSI-2

[F] Arducam UC-444 → [G] Camera A + [H] Camera B → Camera ribbon cables Arducam camera port A → Camera A OV5647 (ribbon cable) Arducam camera port B → Camera B OV5647 (ribbon cable) Only one camera active at a time. Arducam switches between them Both cameras must be same model (OV5647 or both IMX219do not mix)

[A] Pi GPIO 4 → [F] Arducam → Camera select signal GPIO 4 (Pin 7) → Arducam UC-444 SEL pin Function: selects Camera A or Camera B

## Group 7-PWM & Direction GPIO  (TB6612FNG Motor Driver)

| TB6612FNG PIN NOTE | All GPIO assignments below use the RESOLVED pins after conflict resolution. Conflicted original pins are shown in brackets for reference. |
| --- | --- |

| Signal | GPIO (BCM) | Physical Pin | Function | Notes |
| --- | --- | --- | --- | --- |
| ENA-Motor A speed | GPIO 12 / PWM0 | Pin 32 | PWM speed control | Hardware PWM |
| ENB-Motor B speed | GPIO 13 / PWM1 | Pin 33 | PWM speed control | Hardware PWM |
| IN1Motor A dir 1 | GPIO 24 | Pin 18 | Direction pin | RESOLVED (was GPIO 17conflict with UPS HAT) |
| IN2Motor A dir 2 | GPIO 18 | Pin 12 | Direction pin | No conflict |
| IN3Motor B dir 1 | GPIO 26 | Pin 37 | Direction pin | RESOLVED (was GPIO 27conflict with UPS HAT) |
| IN4Motor B dir 2 | GPIO 25 | Pin 22 | Direction pin | No conflict |
| STBY-Standby | 3.3V (Pin 1) | Pin 1 | Tie HIGH | Keep driver enabled |
| VCC Logic supply | 3.3V (Pin 1) | Pin 1 | Logic power | 3.3V from Pi GPIO rail |
| VM Motor supply | External 5V | NOT Pi rail | Motor power | Separate supply coil spikes reset Pi if from Pi rail |
| GND | Pi GND | Pin 6 / Pin 9 | Ground | Shared ground |
| I²C (Grove variant) | GPIO 2/3 | Pins 3/5 | I²C address 0x14 | Via Grove I²C port on Grove Base Hat |

## Group 8-Magnetic Coil Actuator

[N] TB6612FNG → [O] Magnetic Coil → Two output terminals (direct solder) Coil Terminal A → TB6612FNG AO1 (output 1 of channel A) Coil Terminal B → TB6612FNG AO2 (output 2 of channel A) Second coil on second TB6612FNG (if used): Coil Terminal A → BO1 Coil Terminal B → BO2 Coil specs: 25Ω resistance · 200mA at 5V · polarity reversed by swapping IN1/IN2 state Use heat-shrink tubing over soldered connections

## Group 9-Power Chain

[E] Solar Panel → [D] UPS HAT → USB cable (5V charging input) Solar panel 5V USB output → UPS HAT USB charging input Alternative: USB charger (5V/2.5A) → same UPS HAT USB input (bench/indoor use)

[D] UPS HAT → [A] Raspberry Pi → 5V via 40-pin HAT stack UPS HAT stacks on Pi via 40-pin GPIO header Provides stable 5V output to Pi from 18650 cells + USB input I²C monitoring: address 0x42 (Waveshare) / 0x36 (alternate) → GPIO 2/3 Power-fail signal: UPS HAT → GPIO 17 (Pin 11) → Pi [GPIO 17 is now dedicated to UPS HAT onlyNRF24L01 CE moved to GPIO 24] Online status signal: UPS HAT → GPIO 27 (Pin 13) → Pi [GPIO 27 is now dedicated to UPS HAT onlyMotor IN3 moved to GPIO 26]

[A] Pi → All 3.3V devices → 3.3V rail via GPIO Pin 1 Pi GPIO Pin 1 (3.3V) → Grove Base Hat 3.3V rail → all Grove sensors Devices powered: Enviro+ sensors · IMU · GPS · NRF24L01 adapter logic · Arducam logic WARNING: NRF24L01 radio VCC must come from adapter module-NOT directly from this rail

[A] Pi USB → [J] HaLow module → 5V via USB cable Pi USB-A port → HaLow Debug Board → HT-HC01P module Powered from Pi USB 5V output-no separate supply needed

## Group 10IMU Interrupt

[I] MPU-9250/6500 IMU → [A] Pi → Interrupt signal direct GPIO IMU interrupt pin → GPIO 23 (Pin 16) Function: signals Pi when new IMU data is ready to read

# 3. Three-Panel Diagram Specification

| P1 | Panel A-Board 1: Core Stack Components: [A] Pi [B] Grove Base Hat [C] Enviro+ HAT |
| --- | --- |

- Show the physical stack: Pi at bottom, Grove Base Hat on top, Enviro+ on top of that

- Represent the 40-pin header connections as a thick grey/dark bar between each board

- Label the I²C bus line from GPIO 2/3 → into the Enviro+ block, showing all three device addresses: 0x76 (BME280), 0x23 (LTR-559), 0x49 (ADS1015)

- Show the SPI connection from GPIO 8/9/10/11 → Enviro+ LCD. Label: SPI, CS = GPIO 8

- Show the Grove UART port on the Grove Base Hat with a label 'To GPS [M]Group 4'

- Show I²C bus line continuing from Grove Base Hat to Grove I²C ports with labels 'To UPS HAT [D], Arducam [F], IMU [I], Motor Driver [N]'

| P2 | Panel B-Board 2: Power & Imaging Components: [D] UPS HAT [E] Solar [F] Arducam [G] Cam A [H] Cam B [I] IMU |
| --- | --- |

- Show UPS HAT stacked on Pi40-pin bar connection

- Solar panel → USB cable → UPS HAT (label: 5V USB charging)

- UPS HAT → Pi power path. Label: 5V stable output

- UPS HAT I²C monitoring line to I²C bus. Label: I²C 0x42

- UPS HAT power-fail → GPIO 17 (Pin 11). Label: power-fail signal

- UPS HAT online status → GPIO 27 (Pin 13). Label: status signal

- Arducam: CSI ribbon from Pi CSI port. Label: 22-pin FFC MIPI CSI-2

- Arducam: I²C line to I²C bus. Label: I²C 0x70 camera select

- Arducam: GPIO 4 (Pin 7) select signal. Label: camera A/B select

- Camera A + Camera B: ribbon cables to Arducam ports. Label: only one active at a time

- IMU: Grove cable to Grove Base Hat I²C port. Label: I²C 0x68

- IMU interrupt → GPIO 23 (Pin 16). Label: interrupt signal

| P3 | Panel C-Board 3: Comms & Actuation Components: [J] HaLow [K] Antenna [L] NRF24L01 [M] GPS [N] TB6612FNG ×2 [O] Coil |
| --- | --- |

- HaLow: show full pathHT-HC01P → Debug Board → USB cable → Pi USB port. Label all steps. Label as USB (no GPIO).

- HaLow antenna: U.FL on module → pigtail cable → SMA antenna [K]. Label: 915 MHz SMA antenna. Add WARNING: attach before power.

- NRF24L01: show all SPI DuPont wires:

- CSN → GPIO 8 (Pin 24)shared CS with Enviro+ LCD

- CE  → GPIO 24 (Pin 18)RESOLVED pin (NOT GPIO 17)

- SCK → GPIO 11 (Pin 23)

- MOSI → GPIO 10 (Pin 19)

- MISO → GPIO 9 (Pin 21)

- IRQ → GPIO 25 (Pin 22)optional

- VCC → Adapter module 3.3V ONLY. Add warning box: never directly from Pi GPIO rail

- GND → Pin 6

- GPS: Grove cable from Grove Base Hat UART port. Label: GPIO 14 TX → GPS RX / GPIO 15 RX ← GPS TX / 9600 baud

- TB6612FNG: show all DuPont GPIO wires:

- ENA → GPIO 12 / PWM0 (Pin 32)

- ENB → GPIO 13 / PWM1 (Pin 33)

- IN1 → GPIO 24 (Pin 18) [resolved]

- IN2 → GPIO 18 (Pin 12)

- IN3 → GPIO 26 (Pin 37) [resolved]

- IN4 → GPIO 25 (Pin 22)

- STBY → 3.3V Pin 1

- VCC → 3.3V Pin 1

- VM → External 5V supply (separate-show as separate source block, NOT Pi)

- I²C → Grove I²C port (0x14)

- Magnetic Coil: two lines from TB6612FNG AO1 and AO2 to coil terminals A and B. Label: 25Ω · 200mA · 5V

# 4. Colour Code for Connection Lines

| Colour | Connection Type | Used For |
| --- | --- | --- |
| RED | Power5V | Solar → UPS HAT · UPS HAT → Pi · Pi USB → HaLow · VM motor supply |
| DARK RED | Power3.3V | Pi 3.3V rail → all sensors · TB6612FNG VCC · STBY |
| BLACK | Ground (GND) | All ground connections |
| BLUE | I²C bus | SDA (GPIO 2) and SCL (GPIO 3)all I²C connections |
| ORANGE | SPI bus | MOSI (GPIO 10) · MISO (GPIO 9) · SCK (GPIO 11) |
| YELLOW | SPI Chip Select | GPIO 8 (CS for NRF24L01 and Enviro+ LCD) |
| GREEN | UART serial | GPIO 14 TX · GPIO 15 RX → GPS |
| PURPLE | PWM | GPIO 12 ENA · GPIO 13 ENB → TB6612FNG speed |
| GREY | GPIO direction/control | IN1–IN4 · CE · IRQ · power-fail · status signals · camera select |
| TEAL / CYAN | USB | HaLow module → Pi USB port |
| PINK | CSI camera ribbon | Pi CSI port → Arducam UC-444 |
| BROWN | Motor output | TB6612FNG AO1/AO2 → Magnetic Coil terminals |

# 5. GPIO Conflicts

Mark both conflicts on Panel C with a ⚠ symbol and a short note. Show the original pin (struck through) and the resolved pin beside it.

| Conflict | Original Pin | Problem | RESOLVED Assignment |
| --- | --- | --- | --- |
| Conflict 1 | GPIO 17 (Pin 11) | TB6612FNG Motor IN1 AND UPS HAT power-fail signal both assigned here | Motor IN1 → GPIO 24 (Pin 18). GPIO 17 kept for UPS HAT power-fail only. |
| Conflict 2 | GPIO 27 (Pin 13) | TB6612FNG Motor IN3 AND UPS HAT online status signal both assigned here | Motor IN3 → GPIO 26 (Pin 37). GPIO 27 kept for UPS HAT online status only. |

| ALSO NOTE ON DIAGRAM | The NRF24L01 connections table in the source document lists CE → GPIO 17 (Pin 11). This is the pre-conflict-resolution pin it was the original before the GPIO 17 conflict was identified. On the diagram: draw NRF24L01 CE → GPIO 24 (Pin 18) as the correct resolved assignment. Add a footnote: 'CE pin reassigned from GPIO 17 to GPIO 24 per conflict resolution.' |
| --- | --- |

# 6. I²C Address Summary

| I²C Address | Device | Board | Location on diagram |
| --- | --- | --- | --- |
| 0x04 | Grove Base Hat MCU (STM32) | B1 | [B] Grove Base Hat block |
| 0x0C | AK8963 Magnetometer (MPU-9250 only) | B2 | Inside [I] IMU blocknote: not present on MPU-6500 |
| 0x14 | TB6612FNG Motor Driver (Grove I²C variant) | B3 | [N] TB6612FNG block |
| 0x23 | LTR-559 Light/Proximity | B1 | Inside [C] Enviro+ block |
| 0x36 | UPS HAT (alternate variant) | B2 | [D] UPS HAT blocknote: check your specific board |
| 0x42 | UPS HAT INA219 (Waveshare default) | B2 | [D] UPS HAT block |
| 0x49 | ADS1015 ADC (for MICS6814 gas sensors) | B1 | Inside [C] Enviro+ block |
| 0x68 | MPU-9250/6500 IMU (AD0 LOW) | B2 | [I] IMU block |
| 0x70 | Arducam UC-444 Multiplexer | B2 | [F] Arducam block |
| 0x76 | BME280 Temp/Humidity/Pressure | B1 | Inside [C] Enviro+ block |