WiFi HaLow

Technology & Interface Guide

Heltec HT-HC01P Module with Raspberry Pi

IEEE 802.11ah  •  902–928 MHz  •  Sub-1GHz IoT Networking

| Parameter | Value |
| --- | --- |
| Standard | IEEE 802.11ah (WiFi HaLow) |
| Frequency Band | 902–928 MHz (US/AU Region) |
| Module | Heltec HT-HC01P |
| Interface | Mini PCIe → USB |
| Max Data Rate | 32.5 Mbps |
| Range | 1–2 km |
| Antenna | 2.6dBi SMA Male 915MHz 195mm |
| Host Platform | Raspberry Pi + Grove Base Hat |

Revision 1.0  •  2026

# 1. What Is WiFi HaLow?

WiFi HaLow (pronounced 'halo') is the industry name for devices implementing the IEEE 802.11ah wireless standard. Unlike conventional WiFi that operates at 2.4GHz or 5GHz, WiFi HaLow works at sub-1GHz frequencies — specifically 902–928MHz in the US and Australian markets. This fundamental difference in operating frequency gives it radically different performance characteristics that make it ideal for IoT, robotics, drones, and industrial deployments.

The technology trades raw throughput for dramatically extended range, better obstacle penetration, and much lower power consumption — properties that standard WiFi simply cannot achieve at 2.4GHz or 5GHz.

## 1.1  The Simple Analogy

Think of regular WiFi as a sprinter — incredibly fast over short distances but quickly exhausted. WiFi HaLow is the marathon runner — slower peak speed but capable of covering enormous distances using far less energy. For applications that need reliable wireless connectivity across a building, campus, or kilometre-scale outdoor area, HaLow is the superior choice.

## 1.2  Why Lower Frequency Means Better Range

The physics of radio propagation favours lower frequencies for long-range communication in several ways:

- Lower frequencies lose less energy over distance — the signal travels further on the same transmit power

- Long waves bend around obstacles more effectively — a property called diffraction that increases at lower frequencies

- Sub-1GHz signals penetrate walls, floors, concrete, and dense vegetation far better than 2.4GHz

- Less interference from crowded 2.4GHz band (microwave ovens, Bluetooth, other WiFi networks)

## 1.3  WiFi HaLow vs Everything Else

| Technology | Range | Speed | Power | Native IP | Best For |
| --- | --- | --- | --- | --- | --- |
| WiFi (2.4GHz) | 50–100m | Up to 600Mbps+ | High | Yes | Laptops, streaming |
| WiFi HaLow (915MHz) | 1km+ | Up to 32.5Mbps | Very Low | Yes | IoT, drones, robots |
| LoRa | 5–15km | <50Kbps | Extremely Low | No | Tiny sensor packets |
| Bluetooth | 10–30m | 3Mbps | Low | No | Personal devices |
| 4G/LTE | 10km+ | High | Medium | Yes | Mobile broadband |
| Zigbee | 10–100m | 250Kbps | Very Low | No | Smart home mesh |

HaLow's biggest advantage over LoRa is native IP networking. Every HaLow device gets a real IP address and runs standard TCP/UDP protocols — SSH, HTTP, MQTT, RTP video streams — exactly like regular WiFi, just over kilometre-scale distances.

# 2. How WiFi HaLow Works

WiFi HaLow uses the same fundamental architecture as regular WiFi — OFDM modulation, WPA3 security, standard 802.11 association and authentication protocols — but adapted for sub-1GHz operation with specific IoT optimisations.

## 2.1  Network Architecture

A HaLow network requires at least two devices: one Access Point (AP) and one or more Stations (STA). The AP is the hub that all stations connect through, exactly like a home router.

| Role | Function | Configured As |
| --- | --- | --- |
| Access Point (AP) | Hub — manages the network, routes traffic | Ground station Pi |
| Station (STA) | Client — connects to and communicates through the AP | Drone/robot Pi |

After powering both devices, allow 1–3 minutes for the 802.11ah association process to complete — HaLow takes longer to associate than standard WiFi due to the extended channel scanning at sub-1GHz.

## 2.2  Key Technical Features

- OFDM modulation adapted for 1/2/4/8 MHz narrow channels at sub-1GHz

- WPA3 security — same encryption standard as modern WiFi

- MCS0–MCS9 modulation and coding schemes for adaptive rate vs range trade-off

- TWT (Target Wake Time) — devices negotiate wake schedules saving battery dramatically

- BSS Colouring — allows multiple HaLow networks to coexist without interference

- Up to 8,191 devices per Access Point — designed for massive IoT deployments

## 2.3  Frequency Regions

| Region | LoRa/HaLow Band | This Module Compatible? |
| --- | --- | --- |
| USA / Canada | 902–928 MHz (US915) | Yes |
| Australia | 915–928 MHz (AU915) | Yes |
| Europe | 863–870 MHz (EU868) | No — need 868MHz variant |
| Asia (Japan/Korea) | 916.5–927.5 MHz (AS923) | Check local variant |

# 3. Heltec HT-HC01P Module

The HT-HC01P is Heltec Automation's WiFi HaLow module in Mini PCIe form factor. It uses the Morse Micro MM6108IQ chipset — one of the leading 802.11ah implementations — and includes an onboard PA (Power Amplifier), high-linearity LNA (Low Noise Amplifier), T/R switch, and 32MHz crystal oscillator.

## 3.1  Module Specifications

| Parameter | Value |
| --- | --- |
| Chipset | Morse Micro MM6108IQ |
| Standard | IEEE 802.11ah (WiFi HaLow) |
| Power Supply | 3.3V |
| Frequency Band | 902–928 MHz |
| Channel Bandwidth | 1 / 2 / 4 / 8 MHz (configurable) |
| Maximum Data Rate | 32.5 Mbps @ 8MHz channel |
| Transmission Range | 1–2 km line of sight |
| Form Factor | Mini PCIe (52-pin) |
| Dimensions | 50.95 × 30.00 × 3.75 mm |
| Antenna Connector | U.FL (I-PEX) on-module |
| Interface to Host | USB 2.0 via Mini PCIe USB pins |
| Sleep/Hibernate | Low-power hibernate with internal/external wake |
| Security | WPA2/WPA3, IEEE 802.11i |

## 3.2  The Debug Board

The HT-HC01P Debug Board is a companion breakout PCB specifically designed for the HC01P module. It cleanly exposes the USB interface, power supply, and programming/debug connections without requiring jumper wires. Key features:

- Mini PCIe slot to accept the HC01P module — secured with M2 retention screw

- USB output connector — routes Mini PCIe USB pins to standard USB-A port

- Onboard 3.3V regulator — powers module from 5V USB bus automatically

- Debug/UART header — for firmware updates and serial console access

- Power LED and status indicators

- No jumper configuration required — clean, production-ready interface

# 4. Antenna — 2.6dBi 915MHz SMA

## 4.1  Antenna Specifications

| Specification | Value |
| --- | --- |
| Gain | 2.6 dBi |
| Frequency | 902–928 MHz |
| Length | 195 mm |
| Connector | SMA Male (center pin, internal threads) |
| Impedance | 50Ω |
| Polarization | Vertical, omnidirectional |
| Radiation Pattern | 360° horizontal coverage |

## 4.2  SMA vs RP-SMA — Critical Distinction

The single most common wiring mistake with RF antennas is confusing SMA and RP-SMA connectors. They look nearly identical but are NOT interchangeable.

| Connector | Center Contact | Threads | Used For |
| --- | --- | --- | --- |
| SMA Male (this antenna) | CENTER PIN present | Internal | LoRa, GPS, Cellular, HaLow |
| SMA Female (module port) | Center socket | External | Module RF ports |
| RP-SMA Male | Center socket/hole | External | WiFi routers (2.4GHz) |
| RP-SMA Female | Center pin | Internal | WiFi access points |

The HT-HC01P module has a U.FL (I-PEX) micro connector on the PCB — not a full SMA port. A pigtail adapter cable is required to connect the SMA antenna to the module.

## 4.3  Antenna Connection Chain

| Step | Component | Connector Type | Notes |
| --- | --- | --- | --- |
| 1 | 2.6dBi SMA Antenna | SMA Male plug | The antenna itself |
| 2 | Pigtail cable (female end) | SMA Female socket | Screws onto antenna |
| 3 | Pigtail cable (U.FL end) | U.FL snap connector | Snaps onto module |
| 4 | HT-HC01P module | U.FL port on PCB | Press straight down to click |

Required pigtail: SMA Female to U.FL/I-PEX, approximately 15cm length. Use low-loss cable type (RG316 or equivalent). Keep it as short as possible — every centimetre of cable introduces signal loss.

# 5. Complete System Architecture

The following diagram shows how all components connect in a typical ground station / drone or robot deployment:

## 5.1  Ground Station Stack

| Layer | Component | Connection | Notes |
| --- | --- | --- | --- |
| Antenna | 2.6dBi 915MHz SMA | SMA Male → pigtail → U.FL | Mount vertically, away from metal |
| HaLow Module | HT-HC01P | Mini PCIe slot on Debug Board | Secure with M2 screw |
| Debug Board | HT-HC01P Debug Board | USB output cable | Powers + interfaces module |
| USB Adapter | Mini PCIe to USB | USB-A → Pi USB port | Bridges Mini PCIe to Pi |
| Host Computer | Raspberry Pi | 40-pin GPIO header | Runs HaLow network stack |
| Sensors | Grove Base Hat | Stacks on Pi GPIO | All Grove sensors |

## 5.2  Drone / Robot Stack

The drone/robot side uses an identical hardware stack — same module, same debug board, same antenna — configured as a WiFi HaLow Station (STA) instead of Access Point (AP).

| Component | Ground Station Role | Drone/Robot Role |
| --- | --- | --- |
| HT-HC01P Module | Access Point (AP) | Station (STA) |
| IP Address | 192.168.1.1 (static) | 192.168.1.x (DHCP) |
| Routing | Routes traffic to internet/network | Connects through AP |
| Configuration | /etc/hostapd/hostapd.conf | /etc/wpa_supplicant/wpa_supplicant.conf |

# 6. GPIO & Pin Reference

Because the HT-HC01P connects via USB (through the Debug Board and Mini PCIe to USB adapter), it does NOT occupy any GPIO pins on the Raspberry Pi. All 40 GPIO pins remain available for the Grove Base Hat and other peripherals.

## 6.1  Grove Base Hat — I2C Ports (for other sensors)

| Grove Pin | Wire Colour | Signal | GPIO (BCM) | Physical Pin |
| --- | --- | --- | --- | --- |
| Pin 1 | Yellow | SDA | GPIO 2 | Pin 3 |
| Pin 2 | White | SCL | GPIO 3 | Pin 5 |
| Pin 3 | Red | VCC (3.3V) | 3.3V | Pin 1 |
| Pin 4 | Black | GND | GND | Pin 6 |

## 6.2  Grove Base Hat — UART Port (GPS sensor)

| Grove Pin | Wire Colour | Signal | GPIO (BCM) | Physical Pin | Use |
| --- | --- | --- | --- | --- | --- |
| Pin 1 | Yellow | TX (Pi transmits) | GPIO 14 | Pin 8 | Pi TX → GPS RX |
| Pin 2 | White | RX (Pi receives) | GPIO 15 | Pin 10 | GPS TX → Pi RX |
| Pin 3 | Red | VCC (3.3V) | 3.3V | Pin 1 | Power |
| Pin 4 | Black | GND | GND | Pin 6 | Ground |

## 6.3  GPIO Pins NOT Used by HaLow (HT-HC01P)

Unlike the ALFA AHPI7292S HAT which uses SPI and occupies many GPIO pins, the HT-HC01P + Debug Board + USB Adapter combination uses only the Pi's USB bus. Zero GPIO pins are consumed. The complete Grove Base Hat sensor ecosystem operates entirely without conflict.

# 7. Complete Wiring Summary

| From | To | Cable / Method | Critical Notes |
| --- | --- | --- | --- |
| SMA Antenna (Male) | Pigtail SMA Female | Direct screw — finger tight | Do NOT use tools — over-tightening damages SMA |
| Pigtail U.FL end | HT-HC01P U.FL port | Snap straight down | Press perfectly vertical — U.FL is fragile |
| HT-HC01P module | Debug Board Mini PCIe slot | Insert at 30°, press flat, screw M2 | Secure screw prevents poor contact |
| Debug Board USB | Mini PCIe USB Adapter | USB-A to USB-A cable | Standard USB 2.0 cable |
| Mini PCIe USB Adapter | Raspberry Pi USB port | USB-A cable | Use powered hub if Pi USB insufficient |
| External motor battery | L293D / TB6612 VM | Direct terminals | Separate from Pi power |
| Pi GND | Motor driver GND | Jumper wire | Shared ground — CRITICAL |
| Grove Base Hat | Raspberry Pi GPIO | 40-pin HAT press-fit | Align Pin 1 carefully |
| Grove I2C cable | MPU-9250/6500 | Grove 4-pin to any I2C port | Any of 3 I2C ports on hat |
| Grove UART cable | NEO-M8N GPS | Grove 4-pin to UART port | TX/RX are crossed — Pi TX→GPS RX |

# 8. Software Setup

## 8.1  Prerequisites

| Step | Command | Purpose |
| --- | --- | --- |
| Update system | sudo apt-get update && sudo apt-get upgrade -y | Ensure latest packages |
| Install tools | sudo apt-get install -y build-essential git libssl-dev iw net-tools usbutils | Build and network tools |
| Verify USB device | lsusb | Confirm HC01P detected |
| Check interface | ip link show | Confirm wlan0/halow0 created |

## 8.2  Morse Micro Driver Installation

The HT-HC01P uses the Morse Micro MM6108 chipset which requires the Morse Micro Linux driver:

| Step | Command |
| --- | --- |
| Clone driver | git clone https://github.com/MorseMicro/morse_driver.git |
| Build | cd morse_driver && make |
| Install | sudo make install |
| Load module | sudo modprobe morse |
| Auto-load on boot | echo 'morse' ¦ sudo tee -a /etc/modules |

## 8.3  Access Point Configuration (Ground Station)

| Parameter | Value | Notes |
| --- | --- | --- |
| interface | wlan0 | HaLow interface name |
| driver | nl80211 | Linux wireless driver |
| ssid | HaLow_Link | Network name — customise |
| hw_mode | ah | 802.11ah mode — mandatory |
| country_code | US | Set to your region |
| freq | 917 | Center frequency in MHz |
| wpa | 2 | WPA2 security |
| wpa_passphrase | yourpassword | Minimum 8 characters |

## 8.4  Station Configuration (Drone/Robot)

The station Pi connects to the AP using wpa_supplicant. The SSID and password must exactly match the AP configuration. Allow 1–3 minutes after boot for the HaLow association to complete — this is normal and expected behaviour.

## 8.5  Verify Link Quality

| Command | What It Shows |
| --- | --- |
| iwconfig wlan0 | Basic link status, SSID, signal level |
| iw dev wlan0 link | Detailed connection — TX/RX rate, signal dBm |
| iw dev wlan0 station dump | All connected stations (on AP) |
| ping 192.168.1.1 | Round-trip latency to AP |
| iperf3 -c 192.168.1.1 | Throughput benchmark |

# 9. Troubleshooting

| Problem | Likely Cause | Fix |
| --- | --- | --- |
| USB device not detected | Power insufficient or cable fault | Try powered USB hub; try different cable |
| No wlan0/halow0 interface | Driver not installed | Install Morse Micro driver, run sudo modprobe morse |
| Modules won't associate | Normal — HaLow takes time | Wait 1–3 minutes after both devices boot |
| Poor range | Antenna orientation or connection | Ensure antenna vertical; check U.FL fully clicked |
| U.FL connector won't click | Alignment off | Press perfectly straight down — never at an angle |
| SMA antenna won't fit | Wrong connector type | Verify SMA (center pin) vs RP-SMA (center hole) |
| No signal at all | Frequency mismatch | Confirm both devices set to same country/frequency |
| Pi reboots when modules start | Motor current spike | Use separate battery for motors — not Pi USB |
| GPS no NMEA output | Serial console conflict | Disable serial login shell in raspi-config |
| I2C device not found | I2C disabled | Enable I2C in raspi-config, reboot, run i2cdetect -y 1 |

# 10. Key Notes & Best Practices

- Always attach the antenna BEFORE powering the HT-HC01P module — transmitting without an antenna damages the RF frontend

- The HT-HC01P connects via USB only — zero GPIO pins are consumed, all Grove ports remain free

- You need TWO complete HaLow setups for a working link — one AP, one STA

- HaLow association takes 1–3 minutes — this is normal, not a fault

- Keep the SMA-to-U.FL pigtail cable as short as possible to minimise signal loss

- Mount the antenna vertically for best omnidirectional coverage — avoid metal surfaces nearby

- The 915MHz band is US/Australia only — EU users must use the 868MHz variant

- HaLow is native IP — once connected, the drone/robot Pi is on the network exactly like regular WiFi

- Always share GND between Pi, motor drivers, and all power supplies

- U.FL connectors are rated for ~30 mating cycles — minimise plugging and unplugging

# 11. Reference Sources

| Resource | URL |
| --- | --- |
| Heltec HT-HC01P Wiki | https://wiki.heltec.org/docs/devices/wifi-halow/hc01p/ |
| Heltec HT-HC01P Datasheet | https://resource.heltec.cn/download/HT-HC01P/Datasheet/HT-HC01P_Rev.1.0.0.pdf |
| Morse Micro MM6108 SDK | https://github.com/MorseMicro |
| Seeed Grove Base Hat Wiki | https://wiki.seeedstudio.com/Grove_Base_Hat_for_Raspberry_Pi/ |
| WiFi HaLow IEEE 802.11ah Overview | https://www.wi-fi.org/discover-wi-fi/wi-fi-certified-halow |
| SparkFun — SMA Connector Guide | https://learn.sparkfun.com/tutorials/connector-basics/sma-antenna-connectors |
| Raspberry Pi UART Documentation | https://www.raspberrypi.com/documentation/computers/configuration.html#configure-uarts |
| u-blox NEO-M8 Hardware Manual | https://content.u-blox.com/sites/default/files/NEO-M8_HardwareIntegrationManual_(UBX-13003557).pdf |